Georgia Central Railroad

TIMETABLE #4

Effective 0001 hours XX Standard Time

May June 1, 2025

(The graphics team will design the cover

They will need an effective date and time zone)

Georgia Central Railroad

Railroad Address 186 Winge Road Lyons, GA 30436

Timetable Characters & Abbreviations	2

Misc. Contacts and Telephone Numbers	3

Railroad Contacts and Telephone Numbers	3

A Subdivision	4

Timetable Instructions – All Subdivisions	7

| # Timetable Characters & Abbreviations |  |
| --- | --- |
| A | Automatic Interlocking |
| B | Radio Base Station |
| C | Commuter Passenger Station |
| CP | Control Point |
| CS | Controlled Siding |
| CTC | Centralized Traffic Control |
| EMT | End of Main Track |
| FT | Feet |
| G | General Order/Bulletin/SSI Location |
| J | Railroad Junction |
| M | Manual Interlocking |
| MT | Main Track |
| OTM | Other Than Main Track |
| TWC | Track Warrant Controlled |
| W | Wye |
| X | Main Track Crossover |
| Y | Yard |
| YL | Yard Limits |

| # Misc. Contacts and Telephone Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title | Phone | Fax | Cell/Other |
| Emergency/Dispatcher | 866-527-3499 |  | 802-527-3499 |
| ARDC Train Dispatcher | 802-527-3495 | 904-256-1491 | 802-527-4526 |
| Crew Caller's Office | 802-527-3441 |  | 802-527-4558 |
| Dispatcher Manager Desk | 802-527-3590 |  |  |
| Customer Service | 800-757-7387 | 904-256-1473 | MNA-CS@gwrr.com |
| mCrew Support | 904-900-6330 |  |  |
| In case of EMERGENCY, call 911 |  |  |  |

| # Railroad Contacts and Telephone Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Title | Email | Cell |
| Joe Underwood | General Manager | Joseph.underwood@gwrr.com | 904-947-6115 |
| Mitch Klotz | Mechanical Manager | Mitchell.klotz@gwrr.com | 912-293-1266 |
| David Liskey | Roadmaster | David.liskey@gwrr.com | 912-433-7034 |
| Tonya Lamb | Office Manager | Tonya.lamb@gwrr.com | 912-639-2751 |
| Dekota Dobbs | Trainmaster | Dekota.dobbs@gwrr.com | 478-278-5717 |
| Andy Algiers | Trainmaster | Andrew.algiers@gwrr.com | 912-665-1942 |
| James West | Trainmaster | Jwest@gwrr.com | 912-210-7701 |

| # Macon Subdivision |  |  |  |  |
| --- | --- | --- | --- | --- |
| Siding Feet | Mile Post | Station Name | Direction | TT Characters |
|  | 4.7-5.2 | Smithsonia Siding |  | TWC |
|  | 15.6-16.0 | Fitzpatrick Siding | W | TWC |
|  | 23.0-23.2 | Jeffersonville Siding |  | TWC |
|  | 37.4-37.6 | Montrose Siding |  | TWC |
| 6300 | 49.5-50.8 | West Dublin Siding |  | Y |
|  | 57.0-57.4 | East Dublin Yard Tracks 1 & 2 |  | Y |
| 5100 | 81.6-82.0 | Tarrytown Siding |  | TWC |

| Method of Operation |  |  |  | Authorized Speed |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mile Post | Mile Post | Authority | Direction | Mile Post | Mile Post | Speed | Direction |
| 0.0 | 6.0 | Yard Limits |  | 0.0 | 2.0 | 10 |  |
| 6.0 | 49.0 | TWC | W | 2.0 | 6.0 | 20 | W |
| 49.0 | 58.6 | Yard Limits |  | 6.0 | 49.0 | 25 |  |
| 58.6 | 91.3 | TWC |  | 49.0 | 58.6 | 10 |  |
| 91.3 | 92.3 | Yard Limits |  | 58.6 | 91.3 | 25 |  |
|  |  |  |  | 91.3 | 92.3 | 10 |  |

| Radio Procedures |  | FRA-Excepted Track |  |  |
| --- | --- | --- | --- | --- |
| Railroad | Channel | Mile Post | Mile Post | Track Name |
| GC | AAR-004, 004 / Channel 2 |  |  |  |
| NS | AAR-014, 014 / Channel 18 |  |  |  |
| Additional Radio Procedures: |  |  |  |  |

| Defect Detectors |  | Remote Control Operations |  |
| --- | --- | --- | --- |
| Mile Post | Details | Location | Details |
| None |  | None |  |

Equipment Restrictions

- Maximum car weight is 286,000 lbs. based upon a 43’–10” four axle car. Maximum AAR Plate Size is Plate H. - Over-dimensional and/or over-weight loads must be approved by VP-Engineering along with proper clearance documents prior to any movement. - This road is cleared for conventional double-stack container cars meeting the dimensional limitations of the prescribed plate size, including articulated well cars. - Movement of cars with dimensions less than shown, or exceeding the weight limit shown require approval from the Engineering Department.

| Main Track Switch Locations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Station | Location | Switch Name | Direction | Track Number |
|  | 4.7 | WE Smithsonia |  | M |
|  | 5.2 | EE Smithsonia | W | M |
|  | 5.4 | Agile Lead |  | M |
|  | 5.9 | Saddle Creek Lead |  | M |
|  | 15.6 | WE Fitzpatrick |  | M |
|  | 16.0 | EE Fitzpatrick |  | M |
|  | 23.0 | WE Jeffersonville |  | M |
|  | 23.2 | EE Jeffersonville |  | M |
|  | 37.4 | WE Montrose |  | M |
|  | 37.6 | EE Montrose |  | M |
|  | 40.9 | WE Gilman |  | M |
|  | 41.2 | EE Gilman |  | M |
|  | 42.0 | Dudley House Track |  | M |
|  | 49.5 | WE West Dublin |  | M |
|  | 50.8 | EE West Dublin |  | M |
|  | 51.4 | Sunpet |  | M |
|  | 53.5 | WE Dublin Engine Track |  | M |
|  | 53.7 | EE Dublin Engine Track |  | M |
|  | 54.0 | Dublin Construction |  | M |
|  | 57.0 | WE East Dublin |  | M |
|  | 57.2 | Mohawk Lead |  | M |
|  | 57.4 | EE East Dublin |  | M |
|  | 57.4 | Westrock Lead |  | M |
|  | 57.7 | WE Brunswick Chip |  | M |
|  | 58.1 | EE Brunswick Chip |  | M |
|  | 73.9 | Gen Fiber |  | M |
|  | 75.1 | WE Runaround |  | M |
|  | 75.2 | EE Runaround |  | M |
|  | 79.3 | East Coast Asphalt |  | M |
|  | 81.6 | WE Tarrytown |  | M |
|  | 82.0 | EE Tarrytown |  | M |
|  | 91.9 | Wye Track |  | M |
|  | 92.3 | HOG Lead |  | M |

Other Specific Instructions - Macon Subdivision

- Norfolk Southern may operate trains and have MOW personnel working between East Seaboard Junction and West Seaboard Junction switches at any time within Dublin Yard Limits. - Westrock Lead Switch may be left lined for other than main track movement (reversed position). - Dry Branch Lead (MP 9.2) is OUT OF SERVICE – Private locks installed. - Westrock Industry – Use caution walking on clay track and warehouse track due to ground conditions. - Macon Yard – Crossover switch to west end of Track #7 is OUT OF SERVICE – Private locks installed. - MP 8.9 – West-end Run-around track is OUT OF SERVICE – Spiked and tagged with private lock. - Conagra Lead Track in Macon Yard Limits is OUT OF SERVICE – Private locks installed. - All train movements over Highway Grade Crossing listed below has speed restriction as shown: MP 57.5 DOT# 641038R – SR 199 (Westrock Lead) - Eastbound Approach – Maximum Speed 5 MPH - Westbound Approach – Maximum Speed 10 MPH - Montrose Side Track located between MP 37.4 and MP 37.6 is OUT OF SERVICE. Main track switches at East and West Montrose Side Track are spiked and locked with OOS Tags.

Macon Subdivision Track Charts

# Timetable Instructions – All Subdivisions

Arial 10

Notes:________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Railroad Map