Georgia Central Railroad

TIMETABLE #4

Effective 0001 hours XX Standard Time

September 1, 2025

(The graphics team will design the cover

They will need an effective date and time zone)

Georgia Central Railroad

Railroad Address 186 Winge Road Lyons, GA 30436

Timetable Characters & Abbreviations	2

Misc. Contacts and Telephone Numbers	3

Railroad Contacts and Telephone Numbers	3

A Subdivision	4

Timetable Instructions – All Subdivisions	7

| # Timetable Characters & Abbreviations |  |
| --- | --- |
| A | Automatic Interlocking |
| B | Radio Base Station |
| C | Commuter Passenger Station |
| CP | Control Point |
| CS | Controlled Siding |
| CTC | Centralized Traffic Control |
| EMT | End of Main Track |
| FT | Feet |
| G | General Order/Bulletin/SSI Location |
| J | Railroad Junction |
| M | Manual Interlocking |
| MT | Main Track |
| OTM | Other Than Main Track |
| TWC | Track Warrant Controlled |
| W | Wye |
| X | Main Track Crossover |
| Y | Yard |
| YL | Yard Limits |

| # Misc. Contacts and Telephone Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title | Phone | Fax | Cell/Other |
| Emergency/Dispatcher | 866-527-3499 |  | 802-527-3499 |
| ARDC Train Dispatcher | 802-527-3495 | 904-256-1491 | 802-527-4526 |
| Crew Caller's Office | 802-527-3441 |  | 802-527-4558 |
| Dispatcher Manager Desk | 802-527-3590 |  |  |
| Customer Service | 800-757-7387 | 904-256-1473 | MNA-CS@gwrr.com |
| mCrew Support | 904-900-6330 |  |  |
| In case of EMERGENCY, call 911 |  |  |  |

| # Railroad Contacts and Telephone Numbers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Title | Email | Cell |
| Joe Underwood | General Manager | Joseph.underwood@gwrr.com | 904-947-6115 |
| Mitch Klotz | Mechanical Manager | Mitchell.klotz@gwrr.com | 912-293-1266 |
| David Liskey | Roadmaster | David.liskey@gwrr.com | 912-433-7034 |
| Tonya Lamb | Office Manager | Tonya.lamb@gwrr.com | 912-639-2751 |
| Dekota Dobbs | Trainmaster | Dekota.dobbs@gwrr.com | 478-278-5717 |
| Andy Algiers | Trainmaster | Andrew.algiers@gwrr.com | 912-665-1942 |
| James West | Trainmaster | Jwest@gwrr.com | 912-210-7701 |

| # Savannah Subdivision |  |  |  |  |
| --- | --- | --- | --- | --- |
| Siding Feet | Mile Post | Station Name | Direction | TT Characters |
|  | 499.0 | Savannah |  | Y |
|  | 510.2 | Pooler | E | Y |
| 2820 | 513.9 | Meldrim |  | Y |
| 4880 | 519.7 | Ellabell |  | Y |
| 4224 | 535.0 | Groveland |  |  |
|  | 547.3 | Hagan |  |  |
|  | 548.5 | Bellville |  |  |
| 4400 | 556.6 | Collins |  | Y |
|  | 568.0 | Lyons |  |  |
|  | 575.6 | Vidalia |  | Y |

| Method of Operation |  |  |  | Authorized Speed |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mile Post | Mile Post | Authority | Direction | Mile Post | Mile Post | Speed | Direction |
| 499.0 | 521.5 | Yard Limits |  | 499.0 | 504.0 | 10 MPH |  |
| 521.5 | 556.0 | TWC | E | 504.0 | 521.5 | 20 MPH | E |
| 556.0 | 558.0 | Yard Limits |  | 521.5 | 556.0 | 25 MPH |  |
| 558.0 | 575.6 | TWC |  | 556.0 | 558.0 | 10 MPH |  |
| 575.6 | 577.8 | Yard Limits |  | 558.0 | 575.6 | 25 MPH |  |
|  |  |  |  | 575.6 | 577.8 | 10 MPH |  |

| Radio Procedures |  | FRA-Excepted Track |  |  |
| --- | --- | --- | --- | --- |
| Railroad | Channel | Mile Post | Mile Post | Track Name |
| GC | AAR-004, 004 / Channel 2 |  |  | Savannah Yard – See below |
| CSXT | AAR-066, 066 |  |  |  |
|  | AAR-032, 032 / Channel 6 |  |  |  |
| Additional Radio Procedures: |  |  |  |  |

| Defect Detectors |  | Remote Control Operations |  |
| --- | --- | --- | --- |
| Mile Post | Details | Location | Details |
| None |  | None |  |

Equipment Restrictions

- Maximum car weight is 286,000 lbs. based upon a 43’–10” four axle car. Maximum AAR Plate Size is Plate H. - Over-dimensional and/or over-weight loads must be approved by VP-Engineering along with proper clearance documents prior to any movement. - This road is cleared for conventional double-stack container cars meeting the dimensional limitations of the prescribed plate size, including articulated well cars. - Movement of cars with dimensions less than shown or exceeding the weight limit shown require approval from the Engineering Department.

| Main Track Switch Locations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Station | Location | Switch Name | Direction | Track Number |
|  | 501.0 | EE DSI |  | M |
|  | 501.3 | WE DSI | E | M |
|  | 504.1 | Mitsubishi |  | M |
|  | 504.9 | Heniff |  | M |
|  | 507.4 | EE Plastic Express |  | M |
|  | 508.4 | WE Plastic Express |  | M |
|  | 510.2 | EE Pooler Siding |  | M |
|  | 511.15 | Pooler Crossover #1 |  | M |
|  | 511.45 | Pooler Crossover #4 |  | M |
|  | 512.3 | WE Pooler Siding |  | M |
|  | 512.9 | Faulkville Lead |  | M |
|  | 513.9 | EE Meldrim |  | M |
|  | 514.1 | Interfor |  | M |
|  | 514.5 | WE Meldrim |  | M |
|  | 514.8 | SMT Industry |  | M |
|  | 516.6 | EE Hyundai Wye |  | M |
|  | 516.9 | WE Hyundai Wye |  | M |
|  | 517.3 | EE Aggregates USA |  | M |
|  | 518.1 | WE Aggregates USA |  | M |
|  | 519.7 | EE Ellabell |  | M |
|  | 520.7 | WE Ellabell |  | M |
|  | 524.3 | EE Global |  | M |
|  | 524.6 | WE Global |  | M |
|  | 532.6 | Midsouth Aggregates |  | M |
|  | 535.0 | EE Groveland |  | M |
|  | 535.9 | WE Groveland |  | M |
|  | 547.3 | EE Hagan |  | M |
|  | 547.8 | WE Hagan |  | M |
|  | 547.85 | Hagan Industrial |  | M |
|  | 548.5 | EE Bellville |  | M |
|  | 548.9 | WE Bellville |  | M |
|  | 556.6 | EE Collins |  | M |
|  | 557.5 | WE Collins |  | M |
|  | 569.1 | EE Lyons Shop |  | M |
|  | 569.5 | WE Lyons Shop |  | M |
|  | 575.9 | EE Vidalia Track 1 |  | M |
|  | 576.0 | EE Vidalia Yard |  | M |
|  | 576.2 | WE Vidalia Yard |  | M |
|  | 576.3 | WE Vidalia Track 1 |  | M |
|  | 576.3 | Vidalia Engine Track |  | M |
|  | 576.8 | CityLine/Midville |  | M |
|  |  |  |  | M |
|  |  |  |  | M |
|  |  |  |  | M |
|  |  |  |  | M |
|  |  |  |  | M |
|  |  |  |  | M |
|  |  |  |  | M |

Other Specific Instructions - Savannah Subdivision

- All Savannah trains and engines must have in their possession a copy of all current CSX Bulletins. Eastward train and engine movements will be governed by signal indication beginning at Telfair Road and verbal permission to proceed from the CSX Yardmaster and Southover Yard. - All Georgia Central Railway train and engine movements must obtain permission from the CSX Yardmaster at Southover Yard before occupying CSX tracks in Savannah. - CLOSE CLEARANCE – Savannah Yard – Sonoco Paper Track – Use Caution - CLOSE CLEARANCE – MP 501 DSI – Do not ride cars/engines while entering, or inside building. - When setting out railcars on East End of Vidalia Yard Limits a single car set out is not allowed. - Highway Grade Crossing listed below are operating as ISLAND CIRCUIT ONLY. When approaching these crossing, STOP & PROCEED: - Vidalia Yard – DOT# 732634T – Hwy 297/McIntosh Street - MP 577.3 – DOT# 641090V – Brinson Road - Due to the configuration of switches within the signal circuitry, all eastbound train movements approaching Highway Grad Crossings in Vidalia Yard Limits listed below should approach with a maximum speed of 5 MPH. - MP 575.8 – DOT# 641074L – Broadfoot Boulevard - MP 576.7 – DOT# 641083K – Thompson Street - Savannah Yard: All tracks between north and south leg of Wye Track and Magazine Avenue are FRA Excepted tracks. - When driving company vehicles obey all traffic laws while maintaining awareness of your surroundings. When on non-public roadways do not exceed 10 MPH. (railyards, customer facilities, any property near tracks, etc.)

# Timetable Instructions – All Subdivisions

Arial 10

Notes:________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Railroad Map